📄 Technical Brief: ASABE 2026 Intelligent Fruit Sorting System

1. Executive Summary

Objective: Develop a high-performance, open-source C++ Qt application (Objective #4) that acts as the central control unit for an in-field apple sorting machine.

Role of Software: It must ingest high-speed video, detect defects (using Deep Learning), and trigger physical hardware to sort fruit in real-time.

2. Software Architecture (The "OpenWeedGUI" Blueprint)

Based on: 2024 OpenWeedGUI.pdf

Your software structure should mimic the Model-View-Controller (MVC) pattern used in OpenWeedGUI, but adapted for C++.

A. The Frontend (View)

- Framework: Qt Widgets (C++).

- Key Components:

  - Live View: A QLabel or QOpenGLWidget to render the camera feed.

  - Controls: "Start/Stop" buttons, "Sensitivity" sliders (to adjust confidence thresholds), and "Save Data" toggles.

  - Dashboard: Real-time graphs (Qt Charts) showing:

    - Throughput: Apples sorted per minute.

    - Quality stats: % of "Fancy" vs. "Cull" (Defective).

B. The Backend (Controller/Model)

- Threading Strategy (Crucial): You cannot run the AI model on the main GUI thread, or the interface will freeze.

  - Main Thread: Handles button clicks and drawing the image.

  - Worker Thread 1 (Grabber): Pulls raw frames from the camera SDK.

  - Worker Thread 2 (Inference): Runs the Deep Learning model (YOLO) and outputs coordinates.

- Communication: Use Qt Signals & Slots to pass data between threads.

3. Hardware Interface Specs (The "Lu et al." System)

Based on: 2021 & 2022 Apple Sorting & Harvest Assist Papers

Your software acts as the bridge between the Camera and the Ejector.

A. Input: The Multispectral Camera

- The Challenge: The papers reference "Multispectral" (RGB + NIR). This means you aren't just reading a .jpg; you might be reading a multi-channel stream (e.g., 4-channel TIFF or raw binary data).

- Implementation:

  - If standard USB: Use cv::VideoCapture (OpenCV).

  - If Industrial (likely JAI or similar): You must link the GenICam or Manufacturer SDK (e.g., Pylon, Spinnaker) in your C++ CMakeLists.txt.

B. Output: The Sorter Control

- The Mechanism: The sorter uses "pneumatic ejectors" or "fingers."

- The Trigger: Your software decides "Defect detected!": It must send a signal to a PLC (Programmable Logic Controller) or Microcontroller (Teensy/Arduino) to fire the specific ejector.

- Protocol: Likely Serial Communication (UART/RS-232) or Modbus over TCP/IP. You will use the QSerialPort library in Qt.

4. Computer Vision Pipeline (The "Defect" Logic)

Based on: 2022 Ch13 Sorting for Defects & EXPO Papers

The "Defect" isn't just one class. Your model must distinguish types to grade effectively.

Processing Steps:

- ROI Extraction: The camera sees the whole conveyor belt. Your code must "Singulate" (crop) the apple from the black background.

- Multispectral Fusion: If you have separate RGB and NIR streams, you must "register" (align) them so the AI sees both visible color and hidden bruises.

- Inference (The AI):

  - Classes: Normal, Bruise (invisible in RGB), Rot, Cuts/Punctures.

  - Format: The model will output Bounding Boxes [x, y, w, h, class_id].

- Grading Logic: Write a C++ function that maps detections to grades:

  - Logic: If (Bruise_Size > 10mm) OR (Rot detected) -> Grade: CULL.

  - Logic: Else -> Grade: FANCY.

5. Action Plan & Next Steps

Phase 1: The "OpenWeedGUI" Skeleton

- [ ] Action: Create a blank Qt C++ project (using CMake).

- [ ] Action: Build a basic GUI with a "Load Video" button that plays a recorded video file (simulating a camera).

- [ ] Action: Create a QThread that takes frames from that video and runs a dummy process (e.g., converts them to grayscale) to prove you can process without freezing the GUI.

Phase 2: The AI Bridge

- [ ] Action: Export your best YOLOv8 model to ONNX format (yolo export model=best.pt format=onnx).

- [ ] Action: Install ONNX Runtime for C++ or use cv::dnn::readNetFromONNX to load this model in your Qt app.

Phase 3: Hardware "Mocking"

- [ ] Action: Ask Prof. Lu for the specific Camera Model Number and Sorter Controller Protocol.

- [ ] Action: Write a "Mock" class that prints "Signal Sent to PLC!" to the console whenever a defect is found in your video file.

This document covers the "What" and "Why."